[Company]

NETWORK ARCHITECTURE STANDARDS

Classification: Internal   |   Version: [Version]

Last Reviewed: [Date]   |   Document Owner: [Owner]

1  Document Version Control

| Version | Last Modified | Modified By | Document Changes |
| --- | --- | --- | --- |
| 0.1 | [Date] |  | Document first created |

2  Document Contents Page

1   Document Version Control	2

2   Document Contents Page	3

3   Network Architecture Standards	5

3.1   Purpose	5

3.2   Scope	5

3.3   Definitions	5

3.4   Policy Requirements	6

3.5   Roles and Responsibilities	9

3.6   Related Documents	10

4   Policy Compliance	11

5   ISO 27001:2022 Controls Addressed	12

3  Network Architecture Standards

3.1  Purpose

This standard defines security requirements for network design patterns, segmentation, and perimeter controls. It supports the consistent secure design of networks across the organisation.

3.2  Scope

This standard applies to all network design and operational activities, including office networks, data centre networks, cloud virtual networks, and operational technology networks.

3.3  Definitions

Zone: A logical grouping of network resources with shared trust level and security requirements

Perimeter: The boundary between trust zones, typically enforced by firewalls or equivalent controls

Demilitarised Zone (DMZ): A perimeter zone hosting services accessible from less trusted networks

Zero Trust: An approach in which trust is not assumed based on network location and access decisions are made dynamically per request

3.4  Policy Requirements

Zone Architecture

Networks must be segmented into zones with defined trust levels and security requirements

Zones must include at minimum: untrusted (internet), perimeter, internal, restricted, and management

Cloud workloads must be segmented using virtual networks, security groups, and network policies

Perimeter Controls

Perimeter firewalls must be deployed at all boundaries between zones

Default policy at perimeters must be deny-all with explicit allow rules

Inbound services must be exposed through the DMZ with reverse proxy or WAF where applicable

Internal Segmentation

Internal networks must be segmented by function, sensitivity, and risk

Microsegmentation should be used for sensitive workloads

Lateral movement between segments must be controlled by zone boundary enforcement

Management Networks

Management traffic must be separated from production traffic via dedicated networks or VLANs

Out-of-band management must be used for critical infrastructure where feasible

Remote Access

Remote access to internal networks must be brokered through VPN or zero trust access platforms

Remote access must require MFA and device posture checks

Zero Trust Adoption

Adopt zero trust principles for new and refreshed services where supported

Treat the network as untrusted and enforce identity-aware access at the application layer

3.5  Roles and Responsibilities

Network Architect: Designs and reviews networks against this standard

Network Operations: Implements and operates network controls

Information Security Manager: Approves zone definitions and exceptions

3.6  Related Documents

Network Security Policy

Network Segmentation Policy

Network Security Configuration Baseline

4  Policy Compliance

4.1  Compliance Measurement

The information security management team will verify compliance with this policy through various methods including but not limited to periodic internal audits, management reviews, automated compliance reporting tools, and documented evidence reviews. Compliance checks will be conducted at least annually or following any significant change to the environment.

4.2  Exceptions

Any exception to this policy must be submitted in writing to the Information Security Manager, who will assess the risk associated with the exception. Approved exceptions must be documented with a risk acceptance statement signed by the appropriate level of management and reported to the Management Review Team. Exceptions are time-limited and subject to review.

4.3  Non-Compliance

Any employee, contractor, or third-party user found to have violated this policy may be subject to disciplinary action up to and including termination of employment or contract. Violations may also result in civil or criminal prosecution where applicable law permits. Suspected violations should be reported to the Information Security Manager or via the designated reporting channel.

4.4  Continual Improvement

This policy is subject to continual improvement as part of the information security management system. The policy will be reviewed at least annually or following any significant information security incident, regulatory change, or material change to the organisation's operating environment. All revisions are subject to management approval and communicated to relevant stakeholders.

5  ISO 27001:2022 Controls Addressed

This document supports compliance with the following ISO/IEC 27001:2022 Annex A controls:

8.20 - Networks security

8.22 - Segregation of networks

8.21 - Security of network services